Coolflow

# DTX

Safety Data Sheet

Revision: 17/10/2024 Version: 5.0

1

Section 1: Identification of the substance/mixture and of the company/undertaking

## Product identifier

Trade name	Coolflow DTX

## Relevant identified uses of the substance or mixture and uses advised against

Relevant identified uses of the substance or mixture

Non-toxic antifreeze and secondary refrigerant with inhibitor functions designed for use in process cooling, refrigeration and HVAC systems.

Uses advised against	Other

## Details of the supplier of the Safety Data Sheet

Company and address	Europa Way, Swansea West Business Park, SA5 4AJ Fforestfach, Swansea

United Kingdom

Tel: +44 (0) 1792 586800

www.hydratech.co.uk

E-mail	info@hydratech.co.uk

Revision	17/10/2024

SDS Version	5.0

Date of previous version	16/08/2024 (5.0)

## Emergency telephone number

Healthcare professionals: Dial 0344 892 0111 to reach The National Poisons Information Service (NPIS) (24 hour service) General public:

England - Dial 111 to reach NHS 111 (24 hour service) Scotland - Dial 112 to reach NHS 24 (24 hour service)

Wales - Dial 111 or 0845 4647 to reach NHS Direct (24 hour service) See section 4 “First aid measures”.

Emergency telephone number	01792 586800

Opening Hours	08:30 - 17:00

First aid advice number	For emergencies that occur outside of office opening hours that pose a threat to human

health, the environment or require immediate first aid advice call:

+44 (0) 1792 572296

Note	This number is for emergencies only.

2

Section 2: Hazards identification

## Classification of the substance or mixture

Not classified according to Regulation (EC) No. 1272/2008 (CLP) as retained and amended in UK law.

## Label elements

### EC No.

N/A

### Labelling - Regulation (EC) No. 1272/2008 (CLP)

Not classified as hazardous according to CLP Regulation (EC) No. 1272/2008 - no hazard label elements required.

In accordance with Article 9 and Article 12 of Regulation (EC) No. 1272/2008 (CLP), This mixture has been assessed as a mixtures where adequate and reliable scientific information demonstrates the occurrence of synergistic and antagonistic effects that renders the mixture non-toxic as determined by an EPA certified laboratory with an LD50, oral, rat >15,000 mg/kg bw.

### Signal word

Not applicable.

### Hazard statement(s)

Not applicable.

### Precuationary statement(s)

-

### Safety statement(s)

General	-

Prevention	-

Response	-

Storage	-

Disposal	-

### Hazardous substances

ethanediol;ethylene glycol

### Additional labelling

The product contains a biocidal product.

## Other hazards

### Additional warnings

This mixture/product does not contain any substances known to fulfil the criteria for PBT and vPvB classification.

This product does not contain any substances considered to be endocrine disruptors in accordance with the criteria set out in Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2023/707.

3

Section 3: Composition/information on ingredients

## Substances

Not applicable. This product is a mixture.

## Mixtures

Product/substance	ethanediol;ethylene glycol

Identifiers	CAS No.: 107-21-1

EC No.: 203-473-3 UK-REACH:

Index No.: 603-027-00-1

%w/w	80-95%

Classification	Acute Tox. 4, H302

STOT RE 2, H373

Note	[1]

See full text of H-phrases in section 16. Occupational exposure limits are listed in section 8, if these are available.

### Other information

The classification listed for ethylene glycol above is that which is listed, according to Regulation (EC) No. 1272/2008 (CLP) for neat monoethylene glycol. However, the ethylene glycol present in this mixture is rendered non toxic (as determined by an EPA certified laboratory) upon the addition of Hydratech's patented DeTox™ additive.

More detailed information on the detoxification of monoethylene glycol can be found under "further classification and composition comments" in Section 16 of this Safety Data Sheet.

4

Section 4: First aid measures

## Description of first aid measures

General information	In the case of accident: Contact a doctor or casualty department – take the label or this safety

data sheet.

Contact a doctor if in doubt about the injured person’s condition or if the symptoms persist. Never give an unconscious person water or other drink.

Inhalation	In case of discomfort: bring the person into fresh air.

Skin contact	Upon irritation: rinse with water. In the event of continued irritation, seek medical assistance.

Eye contact	Rinse gently with lukewarm water. Remove any contact lenses if this is easy to do. Continue rinsing. In case of persistent eye irritation or discomfort: Seek medical help.

Ingestion	Rinse and flush mouth thoroughly and consume large quantities of water. In case of continued discomfort: seek medical assistance and bring this safety data sheet.

Burns	Not applicable.

## Most important symptoms and effects, both acute and delayed

General Information	The following symptoms are listed in case of exposure to the 100% neat product.

Neurotoxic effects: This product contains organic solvents, which may cause adverse effects to the nervous system. Symptoms of neurotoxicity include: loss of appetite, headache, dizziness, ringing in ears, tingling sensations of skin, sensitivity to the cold, cramps, difficulty in concentrating, tiredness, etc. Repeated exposure to solvents can result in the breaking down of the skin's natural fat layer and may result in an increased absorption potential of other hazardous substances at the area of exposure.

Inhalation	Inhalation of vapours may cause mild irritation of the upper respiratory tract.

Ingestion	May cause discomfort to the stomach if swallowed.

Skin Contact	Prolonged and repeated contact may cause mild irritation of the skin.

Eye Contact	Direct eye contact may cause reddening of the eyes.

## Indication of any immediate medical attention and special treatment needed

Treat symptomatically.

### Information to medics

Bring this safety data sheet or the label from this product.

5

Section 5: Firefighting measures

## Extinguishing media

Suitable extinguishing media: Alcohol-resistant foam, carbon dioxide, powder, water mist. Unsuitable extinguishing media: Waterjets should not be used, since they can spread the fire.

## Special hazards arising from the substance or mixture

Specific hazards	Fire will result in dense smoke. Exposure to combustion products may harm your health.

Closed containers, which are exposed to fire, should be cooled with water. Do not allow fire-extinguishing water to enter the sewage system and nearby surface waters.

Unusual fire and explosion hazards	If the product is exposed to high temperatures, e.g. in the event of fire, dangerous

decomposition compounds are produced. These are: Carbon oxides (CO / CO2)

Fire will result in dense smoke. Exposure to combustion products may harm your health. Closed containers, which are exposed to fire, should be cooled with water. Do not allow fire-extinguishing water to enter the sewage system and nearby surface waters.

If the product is exposed to high temperatures, e.g. in the event of fire, dangerous decomposition compounds are produced. These are: Carbon oxides (CO / CO2)

## Advice for firefighters

No specific requirements.

6

Section 6: Accidental release measures

## Personal precautions, protective equipment and emergency procedures

Ensure adequate ventilation, especially in confined areas. Contaminated areas may be slippery.

## Environmental precautions

Avoid discharge to lakes, streams, sewers, etc. Keep unauthorized persons away from the spill

## Methods and material for containment and cleaning up

Contain and collect spillage with non-combustible, absorbent material e.g. sand, earth, vermiculite or diatomaceous earth and place in container for disposal according to local regulations.

Wherever possible cleaning should be performed with normal cleaning agents. Avoid use of solvents.

## Reference to other sections

See section 13 “Disposal considerations” on handling of waste.

See section 8 "Exposure controls/personal protection" for protective measures.

7

Section 7: Handling and storage

## Precautions for safe handling

Avoid contact during pregnancy and while nursing.

Smoking, drinking and consumption of food is not allowed in the work area.

See section 8 "Exposure controls/personal protection" for information on personal protection.

## Conditions for safe storage, including any incompatibilities

Containers that have been opened must be carefully resealed and kept upright to prevent leakage.

Recommended storage material	Keep only in original packaging.

Storage temperature	0 - 40°C

Dry, cool and well ventilated

Incompatible materials	Reducing agents Bases

Strong acids

Strong oxidizing agents

## Specific end use(s)

This product should only be used for applications quoted in section 1.2.

8

Section 8: Exposure controls/personal protection

| 8.1 Control parameters DNEL |  |  |
| --- | --- | --- |
| ethanediol;ethylene glycol Duration | Route of exposure | DNEL |
| Long term – Systemic effects - General population | Dermal | 53 mg/kg bw/day |
| Long term – Systemic effects - Workers | Dermal | 106 mg/kg bw/day |
| Long term – Local effects - General population | Inhalation | 7 mg/m³ |
| Long term – Local effects - Workers | Inhalation | 35 mg/m³ |
| PNEC |  |  |
| ethanediol;ethylene glycol Route of exposure | Duration of exposure | PNEC |
| Freshwater Freshwater sediment Intermittent release (freshwater) Intermittent release (marine water) Marine water Marine water sediment Sewage treatment plant Soil |  | 10 mg/L 37 mg/kg 10 mg/L 10 mg/L 1 mg/L 3.7 mg/kg 199.5 mg/L 1.53 mg/kg |

## 8.2 Exposure controls

General recommendations	Smoking, drinking and consumption of food is not allowed in the work area.

Exposure scenarios	There are no exposure scenarios implemented for this product.

Exposure limits	Professional users are subjected to the legally set maximum concentrations for occupational exposure. See occupational hygiene limit values above.

Appropriate technical measures	The formation of vapours must be kept at a minimum and below current limit values (see

above). Installation of a local exhaust system if normal air flow in the work room is not sufficient is recommended. Ensure eyewash and emergency showers are clearly marked. Apply standard precautions during use of the product. Avoid inhalation of vapours.

Hygiene measures	Wash hands after use.

### Measures to avoid environmental exposure

No specific requirements.

### Individual protection measures, such as personal protective equipment

Generally	Use only UKCA marked protective equipment.

### Respiratory Equipment

| Type | Class |  | Colour |  | Standards |
| --- | --- | --- | --- | --- | --- |
| No special when used as intended. |  |  |  |  |  |
| Skin protection |  |  |  |  |  |
| Recommended |  | Type/Category |  | Standards |  |

Dedicated work clothing should be -	- worn. Wear a protective suit in the

event of prolonged periods of work with the product.

| Hand protection Material | Glove thickness (mm) | Breakthrough time (min.) | Standards |
| --- | --- | --- | --- |
| Vinyl/PVC | - | > 120 | EN374-3, EN388 |

| Butyl | 0,3 | > 120 | EN374-2, EN374-3, EN388 |
| --- | --- | --- | --- |
| Neoprene (Neoprene) | - | > 120 | EN374-2, EN374-3, EN388, EN407, EN511 |
| Nitrile | 0,2 | > 120 | EN374-2, EN374-3, EN388 |

### Eye protection

Safety glasses with side shields.

EN166

Type	Standards

Other Protection	Wear suitable protective clothing/footwear as protection against splashing or contamination.

9

Section 9: Physical and chemical properties

## Information on basic physical and chemical properties

Physical state	Liquid

Appearance	Clear

Vapour density (air = 1)	No data available

Colour	Colourless (unless dyed according to customer specification)

Odour	Sweet

Odour / Odour threshold	Characteristic, Sweet

pH	-

pH in solution	7.5-9.0 (25%)

Kinematic viscosity	No data available

Density (g/cm³)	1.10-1.15 (20 °C)

Particle characteristics	Does not apply to liquids.

### Phase changes

Melting point/Freezing point (°C)	<-40

Boiling point (°C)	>150

Vapour pressure	13 Pa (25 °C)

Relative vapour density	2.2

Evaporation Rate	No data available.

Flammability / explosion limits	Not applicable.

Relative density	1.10-1.15 (20 °C) Partition coefficient: n-octanol / water	No data available. Decomposition temperature (°C)	No data available

### Data on fire and explosion hazards

Flash point (°C)	119

Test method: EN ISO 2719

Explosive properties	Not applicable.

Flammability (°C)	The material is not combustible.

Oxidising properties	Not applicable.

Auto-ignition temperature (°C)	>400

Lower and upper explosion limit (% v/v)	Not applicable - based on structure

### Solubility

Solubility in water	Completely soluble

n-octanol/water coefficient	Testing not relevant or not possible due to the nature of the product. Solubility in fat (g/L)	No data available

## Other information

Oxidizing properties	Not applicable - based on structure Other physical and chemical parameters	No data available.

10

Section 10: Stability and reactivity

## Reactivity

No data available.

## Chemical stability

The product is stable under the conditions, noted in section 7 “Handling and storage”.

## Possibility of hazardous reactions

None known.

## Conditions to avoid

Do not expose to any forms of heat (e.g. solar radiation). May lead to excess pressure. Sunlight

Extremes of temperature

## Incompatible materials

Bases

Reducing agents Strong acids

Strong oxidizing agents

## Hazardous decomposition products

Under normal conditions of storage and use, hazardous decomposition products should not be produced

11

Section 11: Toxicological information

## Information on toxicological effects

### Acute toxicity

Based on available data, the classification criteria are not met.

### Skin corrosion/irritation

Based on available data, the classification criteria are not met.

### Serious eye damage/irritation

Based on available data, the classification criteria are not met.

### Respiratory sensitisation

Based on available data, the classification criteria are not met.

### Skin sensitisation

Based on available data, the classification criteria are not met.

### Germ cell mutagenicity

Based on available data, the classification criteria are not met.

### Carcinogenicity

Based on available data, the classification criteria are not met.

### Reproductive toxicity

Based on available data, the classification criteria are not met.

### STOT-single exposure

Based on available data, the classification criteria are not met.

### STOT-repeated exposure

Based on available data, the classification criteria are not met.

### Aspiration hazard

Based on available data, the classification criteria are not met.

## Information on other hazards

Long term effects	Neurotoxic effects: This product contains organic solvents, which may cause adverse effects to the nervous system. Symptoms of neurotoxicity include: loss of appetite, headache, dizziness, ringing in ears, tingling sensations of skin, sensitivity to the cold, cramps, difficulty in concentrating, tiredness, etc. Repeated exposure to solvents can result in the breaking down of the skin's natural fat layer and may result in an increased absorption potential of other hazardous substances at the area of exposure.

Endocrine disrupting properties	This mixture/product does not contain any substances known to have hormone-disrupting

properties in relation to health.

Other information	None known.

12

Section 12: Ecological information

## Toxicity

No data available.

## Persistence and degradability

Based on available data, the classification criteria are not met.

## Bioaccumulative potential

Based on available data, the classification criteria are not met.

## Mobility in soil

No data available.

## Results of PBT and vPvB assessment

This mixture/product does not contain any substances known to fulfil the criteria for PBT and vPvB classification.

## Endocrine disrupting properties

This mixture/product does not contain any substances considered to have endocrine-disrupting properties in relation to the environment.

## Other adverse effects

This product contains substances that are toxic to the environment. May result in adverse effects to aquatic organisms.

13

Section 13: Disposal considerations

## Waste treatment methods

Product is not covered by regulations on dangerous waste.

Regulation (EU) No 1357/2014 of 18 December 2014 on waste as retained and amended in UK law.

### EWC code

16 01 15

Antifreeze fluids other than those mentioned in 16 01 14

### Contaminated packing EWC code

16 01 15

Antifreeze fluids other than those mentioned in 16 01 14

14

Section 14: Transport information

## UN number

Product not classified as hazardous for transport.

ADR	-

IMDG	-

IATA	-

## UN proper shipping name

Product not classified as hazardous for transport.

ADR	-

IMDG	-

IATA	-

## Hazard class(es)

Product not classified as hazardous for transport.

ADR

-

IMDG

-

IATA

-

## Packing group

Product not classified as hazardous for transport.

ADR	-

IMDG	-

IATA	-

## Environmental hazards

Product not classified as an environmental hazard.

| ADR IMDG IATA | - - - |
| --- | --- |
| Other information: ADR | - |
| IMDG | - |
| IATA | - |

### Additional information

Not dangerous goods according to ADR, IATA and IMDG.

## Special precautions for user

Not applicable.

## Maritime transport in bulk according to IMO instruments

No data available.

15

Section 15: Regulatory information

## Safety, health and environmental regulations/legislation specific for the substance or mixture

### Restrictions for application

Pregnant women and women breastfeeding must not be exposed to this product. The risk, and possible technical precautions or design of the workplace needed to eliminate exposure, must be considered.

### Demands for specific education

No specific requirements.

### SEVESO - Categories / dangerous substances

Not applicable.

Additional information	Not applicable.

### Sources

The Health and Safety at Work etc. Act 1974 Regulations 2013.

In accordance with Regulation (EU) No 528/2012 concerning the making available on the market and use of biocidal products as retained and amended in UK law.

Regulation (EU) No 1357/2014 of 18 December 2014 on waste as retained and amended in UK law.

Regulation (EC) No 1272/2008 on classification, labelling and packaging of substances and mixtures (CLP) as retained and amended in UK law.

Regulation (EC) No 1907/2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH) as retained and amended in UK law.

## Chemical safety assessment

No

16

Section 16: Other information

Issued by	Hydratech

### Full text of H-phrases as mentioned in section 3

H302, Harmful if swallowed.

H373, May cause damage to organs through prolonged or repeated exposure.

Revision date	30/03/2023

### Abbreviations and acronyms

ADN = European Provisions concerning the International Carriage of Dangerous Goods by Inland Waterway ADR = The European Agreement concerning the International Carriage of Dangerous Goods by Road

ATE = Acute Toxicity Estimate BCF = Bioconcentration Factor CAS = Chemical Abstracts Service

CE = Conformité Européenne (European conformity)

CLP = Classification, Labelling and Packaging Regulation [Regulation (EC) No. 1272/2008] CSA = Chemical Safety Assessment

CSR = Chemical Safety Report DMEL = Derived Minimal Effect Level DNEL = Derived No Effect Level

EINECS = European Inventory of Existing Commercial chemical Substances ES = Exposure Scenario

EUH statement = CLP-specific Hazard statement EuPCS = European Product Categorisation System EWC = European Waste Catalogue

GHS = Globally Harmonized System of Classification and Labelling of Chemicals GWP = Global warming potential

IARC = International Agency for Research on Cancer (IARC) IATA = International Air Transport Association

IBC = Intermediate Bulk Container

IMDG = International Maritime Dangerous Goods

LogPow = logarithm of the octanol/water partition coefficient

MARPOL = International Convention for the Prevention of Pollution From Ships, 1973 as modified by the Protocol of 1978. ("Marpol" = marine pollution)

OECD = Organisation for Economic Co-operation and Development PBT = Persistent, Bioaccumulative and Toxic

PNEC = Predicted No Effect Concentration

RID = The Regulations concerning the International Carriage of Dangerous Goods by Rail RRN = REACH Registration Number

SCL = A specific concentration limit

SVHC = Substances of Very High Concern

STOT-RE = Specific Target Organ Toxicity - Repeated Exposure STOT-SE = Specific Target Organ Toxicity - Single Exposure TWA = Time weighted average

UN = United Nations

UVBC = Unknown or variable composition, complex reaction products or of biological materials VOC = Volatile Organic Compound

vPvB = Very Persistent and Very Bioaccumulative

Approved by	Dylan Redwood

### Additional information

As an individual chemical compound, Ethylene Glycol (aka Mono Ethylene Glycol) is classified as Acute Tox. 4 - H302 and STOT RE 2 - H373, according to Regulation (EC) No. 1272/2008 and information provided in the REACH registration of this substance.

However, it should be noted that Ethylene Glycol is not independently toxic. Specifically, the above classifications arise from the fact that, upon its metabolization in the liver by the enzyme Alcohol Dehydrogenase (ADH), Ethylene Glycol is ultimately converted into Glycolic Acid and Oxalic Acid. These acids are responsible for the common symptoms often observed in cases of Ethylene Glycol poisoning, including Metabolic Acidosis. Additionally, the Oxalic Acid produced will bind to Serum Calcium in the blood to form Calcium Oxalate Crystals that deposit in the kidneys, causing acute kidney (renal) failure. It has been utilised a patented method of inhibiting the metabolization of Ethylene Glycol in the liver, through the addition of a detoxifier additive.

This additive has been scientifically proven, by an EPA approved laboratory, to inhibit the formation of Glycolic Acid and Oxalic Acid, preventing Ethylene Glycol poisoning, including the deposition of Calcium Oxalate crystals in the kidneys. The test studies by Toxicon, Inc. determined that by the occurrence of synergistic and antagonistic effects the addition of the additive to Ethylene Glycol detoxifies it, resulting in a mixture with an LD50 , Oral, Rat >15,000 mg/kg bw. By raising the LD50 to a level outside of classification as acutely toxic and preventing the targeting of the kidney by Calcium Oxalate Crystals, this mixture does not fall within the limits for classification as hazardous, as either Acute Tox. 4 - H302 or STOT RE 2 – H373.

Revision comments	First edition SDS

### The safety data sheet is validated by

Dylan Redwood (Liquitherm Technologies Group Limited)

### Hazard statements in full

H302, Harmful if swallowed.

H373, May cause damage to organs through prolonged or repeated exposure.

### Other

A change (in proportion to the last essential change (first cipher in SDS version, see section 1)) is marked with a triangle.

The information in this safety data sheet applies only to this specific product (mentioned in section 1) and is not necessarily correct for use with other chemicals/products.

It is recommended to hand over this safety data sheet to the actual user of the product. Information in this safety data sheet cannot be used as a product specification.

Country-language: GB-en

### Further classification and composition comments

As an individual chemical compound, Ethylene Glycol (aka Mono Ethylene Glycol) is classified as Acute Tox. 4 - H302 and STOT RE 2 - H373, according to Regulation (EC) No. 1272/2008 and information provided in the REACH registration of this substance.

However, it should be noted that Ethylene Glycol is not independently toxic. Specifically, the above classifications arise from the fact that, upon its metabolization in the liver by the enzyme Alcohol Dehydrogenase (ADH), Ethylene Glycol is ultimately converted into Glycolic Acid and Oxalic Acid. These acids are responsible for the common symptoms often observed in cases of Ethylene Glycol poisoning, including Metabolic Acidosis. Additionally, the Oxalic Acid produced will bind to Serum Calcium in the blood to form Calcium Oxalate Crystals that deposit in the kidneys, causing acute kidney (renal) failure. It has been utilised a patented method of inhibiting the metabolization of Ethylene Glycol in the liver, through the addition of a detoxifier additive.

This additive has been scientifically proven, by an EPA approved laboratory , to inhibit the formation of Glycolic Acid and Oxalic Acid, preventing Ethylene Glycol poisoning, including the deposition of Calcium Oxalate crystals in the kidneys. The test studies by Toxicon, Inc. determined that by the occurrence of synergistic and antagonistic effects the addition of the additive to Ethylene Glycol detoxifies it, resulting in a mixture with an LD50 , Oral, Rat >15,000 mg/kg bw. By raising the LD50 to a level outside of classification as acutely toxic and preventing the targeting of the kidney by Calcium Oxalate Crystals, this mixture does not fall within the limits for classification as hazardous, as either Acute Tox. 4 - H302 or STOT RE 2 – H373.

### Abbreviations and acronyms

ADN = European Provisions concerning the International Carriage of Dangerous Goods by Inland Waterway ADR = The European Agreement concerning the International Carriage of Dangerous Goods by Road

ATE = Acute Toxicity Estimate BCF = Bioconcentration Factor CAS = Chemical Abstracts Service CE = Conformité Européenne

CLP = Classification, Labelling and Packaging Regulation [Regulation (EC) No. 1272/2008] CSA = Chemical Safety Assessment

CSR = Chemical Safety Report DMEL = Derived Minimal Effect Level DNEL = Derived No Effect Level

EINECS = European Inventory of Existing Commercial chemical Substances ES = Exposure Scenario

EUH statement = CLP-specific Hazard statement EWC = European Waste Catalogue

GHS = Globally Harmonized System of Classification and Labelling of Chemicals IARC = International Agency for Research on Cancer (IARC)

IATA = International Air Transport Association IBC = Intermediate Bulk Container

IMDG = International Maritime Dangerous Goods

LogPow = logarithm of the octanol/water partition coefficient

MARPOL = International Convention for the Prevention of Pollution From Ships, 1973 as modified by the Protocol of 1978. ("Marpol" = marine pollution)

OECD = Organisation for Economic Co-operation and Development PBT = Persistent, Bioaccumulative and Toxic

PNEC = Predicted No Effect Concentration

RID = The Regulations concerning the International Carriage of Dangerous Goods by Rail RRN = REACH Registration Number

SCL = A specific concentration limit

SVHC = Substances of Very High Concern

STOT-RE = Specific Target Organ Toxicity - Repeated Exposure STOT-SE = Specific Target Organ Toxicity - Single Exposure TWA = Time weighted average

UN = United Nations

UVBC = Unknown or variable composition, complex reaction products or of biological materials VOC = Volatile Organic Compound

vPvB = Very Persistent and Very Bioaccumulative

### Training advice

Product should only be handled by trained operators.

### Additional information

In accordance with UK-REACH, a safety data sheet is not required for this product. This safety data sheet has been created on a voluntary basis to distribute relevant information as required by UK-REACH.

The information in this safety data sheet applies only to this specific product (mentioned in section 1) and is not necessarily correct for use with other chemicals/products.

It is recommended to hand over this safety data sheet to the actual user of the product. Information in this safety data sheet cannot be used as a product specification.

Country-language: GB-en